Practical No. 01

Aim: Create network with three routers with RIPv2 and each router associated network will have minimum three PC. Show connectivity.

Steps to perform practical:

Step 1 : Add 3 routers, 3 switches, end user clients to the switches.

Step 2 : Connect all the devices with straight through cable.

Step 3 : Add serial port devices in the routers in the router.

Step 4 : Give ip to each port of router

enable

configure terminal

interface gig0/0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

Step 5 : Set IP address, Subnet & Gateway for End User Client (manually in input boxes). Note that the gateway will be same as the routers IP.

Step 6 : Apply RIP to each of the router

configure terminal

router rip

network 192.168.10.0

network 192.168.40.0

exit

Step 7 : Go for (show ip route) command on each router.

Step 8 : Show connection between PC6 and PC0.

System > Desktop tab > Command Prompt > ping 192.168.10.2

NETWORK TOPOLOGY:

i)Router Configuration:-

Set IP addresses to routers :-

Set IP addresses to all remaining routers as shown above.

ii)End User Client Configuration :

a)Set IP address to end user client :

Set IP addresses to all remaining clients as shown above.

b)Set Gateway to end user client:

Set Gateways to all remaining end user client.

Set RIP protocol to routers :

Set RIP protocol to all remaining routers as shown above.

Go for “show ip route” command for each router:

Show  “show ip route” command for all remaining routers.

Show Packet Tracing :

Show Connection between PC0 to PC6:

FINAL TOPOLOGY:

Conclusion: We have learnt to create RIPv2 with three routers as shown above.

Practical No. 02

Aim: Create network with three routers with OSPF and each router associated network will have minimum three PC. Show connectivity.

Steps to perform practical:

Step 1 : Add 3 routers, 3 switches, end user clients to the switches.

Step 2 : Connect all the devices with straight through cable.

Step 3 : Add serial port devices in the routers in the router.

Step 4 : Give IP to each port of router

enable

configure terminal

interface gig0/0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

Step 5 : Set IP address, Subnet & Gateway for End User Client (manually in input boxes). Note that the gateway will be same as the routers IP.

Step 6 : Apply OSPF to each of the router

configure terminal

router ospf 100

network 192.168.10.0 0.255.255.255 area 0

network 192.168.40.0 0.255.255.255 area 0

exit

Step 7 : Go for (show ip route) command on each router.

Step 8 : Show connection between PC6 and PC0.

System > Desktop tab > Command Prompt > ping 192.168.10.2

NETWORK TOPOLOGY:

i)Router Configuration:-

Set IP addresses to routers :-

Set IP addresses to all remaining routers as shown above.

ii)End User Client Configuration :

a)Set IP address to end user client :

Set IP addresses to all remaining clients as shown above.

b)Set Gateway to end user client:

Set Gateways to all remaining end user client.

Set OSPF protocol to routers :

Set OSPF protocol to all remaining routers as shown above.

Go for “show ip route” command for each router:

Show  “show ip route” command for all remaining routers.

Show Packet Tracing :

Show Connection between PC0 to PC6:

FINAL TOPOLOGY:

Conclusion:- We have learnt to create OSPF with three routers as shown above.

Practical No. 03

Aim: Create network with three routers with BGP and each router associated network will have minimum three PC. Show connectivity.

Steps to perform practical:

Step 1 : Add 3 routers, 3 switches, end user clients to the switches.

Step 2 : Connect all the devices with straight through cable.

Step 3 : Add serial port devices in the routers in the router.

Step 4 : Give IP to each port of router

enable

configure terminal

interface gig0/0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

Step 5 : Set IP address, Subnet & Gateway for End User Client (manually in input boxes). Note that the gateway will be same as the routers IP.

Step 6 : Apply OSPF to each of the router

configure terminal

router bgp 100

neighbor 192.168.40.2 remote-as 200

network 192.168.10.0

network 192.168.40.0

exit

Step 7 : Go for (show ip route) command on each router.

Step 8 : Show connection between PC6 and PC0.

System > Desktop tab > Command Prompt > ping 192.168.10.2

NETWORK TOPOLOGY:

i)Router Configuration:-

Set IP addresses to routers :-

Set IP addresses to all remaining routers as shown above.

ii)End User Client Configuration :

a)Set IP address to end user client :

Set IP addresses to all remaining clients as shown above.

b)Set Gateway to end user client:

Set Gateways to all remaining end user client.

Set BGP protocol to routers :

Set BGP protocol to all remaining routers as shown above.

Go for “show ip route” command for each router:

Show  “show ip route” command for all remaining routers.

Show Packet Tracing :

Show Connection between PC0 to PC6:

FINAL TOPOLOGY:

Conclusion:- We have learnt to create BGP with three routers as shown above.

Practical No. 04

Aim: Configure DHCP server and client for DHCP service.

Steps to perform practical:

Step 1 : Add  router, switch and end user clients to the switches.

Step 2 : Connect all the devices with straight through cable.

Step 3 : Give IP to each port of router

enable

configure terminal

interface gig0/0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

Step 4 : Apply DHCP to the router

configure terminal

ip dhcp pool pool_name

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

exit

ip dhcp excluded-address 192.168.10.2 192.168.10.10

exit

Step 5 : Show DHCP ip address to each end user clients.

System > Desktop tab > ip configuration > select dhcp.

DHCP automatically assign ip address to each client.

NETWORK TOPOLOGY:

i)Router Configuration:-

Set IP addresses to routers :-

ii)End User Client Configuration :

a)Set Gateway to end user client:

Set Gateways to all remaining end user client.

Set DHCP protocol to routers :

Show DHCP ip address to each end user clients.

Show  DHCP ip address to all remaining end user clients as shown above.

FINAL TOPOLOGY:

Conclusion:- We have learnt to create DHCP server with client service as shown above.

Practical No. 05

Aim: Create simple Adhoc network.

Installation Steps:

Step 1: Go to omnetpp-5.5.1 folder in which open “mingwenv” file, we get following window. Type “./configure” command.

Step 2: After successfully run above command, type another command “make”.

Practical Steps:

Step 1: Go to omnetpp-5.5.1 folder in which open “mingwenv” file, we get following window. Type “omnetpp” command to open omnet++ IDE.

Step 2: after that command, following window will open.

Step 3: Go to project Explorer > inet > examples > Adhoc > Hostautoconf  and open Scenario.ned file.

Scenario.ned:

Coding:

Scenario.ned:

package inet.examples.adhoc.hostautoconf;

import inet.physicallayer.ieee80211.packetlevel.Ieee80211ScalarRadioMedium;

import inet.visualizer.contract.IIntegratedVisualizer;

network Scenario

{

parameters:

int hosts;

@display("bgb=650,450");

submodules:

visualizer: <default("IntegratedCanvasVisualizer")> like IIntegratedVisualizer if hasVisualizer() {

parameters:

@display("p=100,200;is=s");

}

radioMedium: Ieee80211ScalarRadioMedium {

parameters:

@display("p=100,100;is=s");

}

host[hosts]: Host {

@display("p=300,150");

}

}

Omnetpp.ini:

[General]

#debug-on-errors = true

#record-eventlog = true

network = Scenario

sim-time-limit = 60min

cmdenv-express-mode = true

*.hosts = 3

**.constraintAreaMinX = 0m

**.constraintAreaMinY = 0m

**.constraintAreaMinZ = 0m

**.constraintAreaMaxX = 600m

**.constraintAreaMaxY = 400m

**.constraintAreaMaxZ = 0m

# mobility

**.host*.mobility.typename = "MassMobility"

**.host*.mobility.initFromDisplayString = false

**.host*.mobility.changeInterval = truncnormal(2s, 0.5s)

**.host*.mobility.angleDelta = normal(0deg, 30deg)

**.host*.mobility.speed = truncnormal(20mps, 8mps)

**.host*.mobility.updateInterval = 100ms

**.host*.ac_wlan.interfaces = "wlan0"

# UdpBasicApp / UdpSink

**.host*.numApps = 1

**.app[0].typename = "UdpBasicApp"

**.app[0].destAddresses = "host[0]"

**.app[0].localPort = 9001

**.app[0].destPort = 9001

**.app[0].messageLength = 100B

**.app[0].startTime = uniform(10s, 30s)

**.app[0].sendInterval = uniform(10s, 30s)

# nic settings

**.wlan[*].bitrate = 2Mbps

**.wlan[*].mgmt.frameCapacity = 10

**.wlan[*].mac.retryLimit = 7

**.wlan[*].mac.dcf.channelAccess.cwMin = 7

**.wlan[*].mac.cwMinBroadcast = 31

**.wlan[*].radio.transmitter.power = 2mW

**.wlan[*].radio.receiver.sensitivity = -85dBm

**.wlan[*].radio.receiver.snirThreshold = 4dB

**.udpapp.*.vector-recording = true

**.vector-recording = true

Step 4: Click on Run button.

Step 5: After that following window will open.

Step 6:Click on RUN.

OUTPUT:

Conclusion: We have learnt to create a simple ADHOC network.

Practical No. 06

Aim: Create MANET simulation for AODVUU Network.

Step 1: Go to omnetpp-5.5.1 folder in which open “mingwenv” file, we get following window. Type “omnetpp” command to open omnet++ IDE.

Step 2: after that command, following window will open.

Step 3: Go to project Explorer > inet > examples > aodv and open AODVNetwork.ned file.

AODVNetwork.ned:

Coding:

AODVNetwork.ned:

package inet.examples.aodv;

import inet.common.scenario.ScenarioManager;

import inet.networklayer.configurator.ipv4.Ipv4NetworkConfigurator;

import inet.networklayer.ipv4.RoutingTableRecorder;

import inet.node.aodv.AodvRouter;

import inet.physicallayer.unitdisk.UnitDiskRadioMedium;

network AODVNetwork

{

parameters:

int numHosts;

@display("bgb=650,650");

submodules:

radioMedium: UnitDiskRadioMedium {

parameters:

@display("p=100,200;is=s");

}

configurator: Ipv4NetworkConfigurator {

parameters:

config = xml("<config><interface hosts='*' address='145.236.x.x' netmask='255.255.0.0'/></config>");

@display("p=100,100;is=s");

}

routingTableRecorder: RoutingTableRecorder {

parameters:

@display("p=100,300;is=s");

}

scenarioManager: ScenarioManager {

parameters:

script = default(xml("<scenario/>"));

@display("p=100,400;is=s");

}

host[numHosts]: AodvRouter {

parameters:

@display("i=device/pocketpc_s;r=,,#707070");

}

connections allowunconnected:

}

Omnetpp.ini:

[General]

network = AODVNetwork

#record-eventlog = true

num-rngs = 3

debug-on-errors = true

**.mobility.rng-0 = 1

**.wlan[*].mac.rng-0 = 2

# channel physical parameters

**.wlan[*].typename = "AckingWirelessInterface"

**.wlan[*].bitrate = 2Mbps

**.wlan[*].mac.headerLength = 20B

**.wlan[*].radio.typename = "UnitDiskRadio"

**.wlan[*].radio.transmitter.headerLength = 96b

**.wlan[*].radio.transmitter.communicationRange = 250m

**.wlan[*].radio.transmitter.interferenceRange = 0m

**.wlan[*].radio.transmitter.detectionRange = 0m

**.wlan[*].radio.receiver.ignoreInterference = true

*.numHosts = 20

# mobility

**.host[*].mobility.typename = "StationaryMobility"

**.mobility.constraintAreaMinZ = 0m

**.mobility.constraintAreaMaxZ = 0m

**.mobility.constraintAreaMinX = 0m

**.mobility.constraintAreaMinY = 0m

**.mobility.constraintAreaMaxX = 600m

**.mobility.constraintAreaMaxY = 600m

# ping app (host[0] pinged by others)

*.host[0].numApps = 1

*.host[0].app[0].typename = "PingApp"

*.host[0].app[0].startTime = uniform(1s,5s)

*.host[0].app[0].printPing = true

# nic settings

**.wlan[*].bitrate = 2Mbps

**.wlan[*].mgmt.frameCapacity = 10

**.wlan[*].mac.retryLimit = 7

# lifecycle

**.hasStatus = true

[Config Static]

description = routing without mobility

*.host[*].wlan[*].radio.transmitter.communicationRange = 250m

*.host[0].app[0].destAddr = "host[1](ipv4)"

[Config IPv4SlowMobility]

description = two fixed communicating nodes with low speed mobile nodes

extends = Static

# mobility

**.aodv.activeRouteTimeout = 3s

**.host[2..20].mobility.typename = "MassMobility"

**.host[0].mobility.typename = "StationaryMobility"

**.host[1].mobility.typename = "StationaryMobility"

**.host[*].mobility.changeInterval = normal(5s, 0.1s)

**.host[*].mobility.angleDelta = normal(0deg, 30deg)

**.host[*].mobility.speed = normal(2mps, 0.01mps)

**.host[1].mobility.initialX = 600m

**.host[1].mobility.initialY = 600m

[Config IPv4ModerateFastMobility]

description = two fixed communicating nodes with moderate speed mobile nodes

extends = IPv4SlowMobility

# mobility

**.aodv.activeRouteTimeout = 2s

**.host[*].mobility.speed = normal(8mps, 0.01mps)

[Config IPv4FastMobility]

description = two fixed communicating nodes with high speed mobile nodes

extends = IPv4SlowMobility

# mobility

**.aodv.activeRouteTimeout = 1s

**.host[*].mobility.speed = normal(15mps, 0.01mps)

[Config Dynamic]

description = one node is shut down and restarted trigger route changes

extends = Static

*.host[*].hasStatus = true

*.scenarioManager.script = xmldoc("dynamic.xml")

[Config MoreDynamic]

description = some nodes are shut down trigger route changes

extends = Static

*.host[*].hasStatus = true

*.scenarioManager.script = xmldoc("moreDynamic.xml")

[Config SimpleRREQ]

description = demonstrates a single RREQ-RREP exchange

network = SimpleRREQ

# nic settings

**.wlan[*].radio.transmitter.communicationRange = 240m

**.sender.numApps = 1

**.sender.app[0].typename = "PingApp"

**.sender.app[0].startTime = uniform(1s,5s)

**.sender.app[0].printPing = true

**.sender.app[0].destAddr = "receiver(ipv4)"

[Config SimpleRREQ2]

description = demonstrates a single RREQ-RREP exchange with two intermediate nodes

extends = SimpleRREQ

network = SimpleRREQ2

[Config SimpleLifecycle]

description = demonstrates AODV's RERR mechanism when a node shuts down

extends = SimpleRREQ2

*.scenarioManager.script = xmldoc("simpleLifecycle.xml")

[Config ShortestPath]

description = demonstrates that AODV chooses the shorter path

network = ShortestPath

extends = SimpleRREQ

Step 4: Click on Run button.

Step 5: After that following window will open.

Step 6:Click on RUN.

OUTPUT:

Conclusion: We have learnt to MANET simulation for AODVUU Network.

Practical No. 07

Aim: Create Single mobile network.

Step 1: Go to omnetpp-5.5.1 folder in which open “mingwenv” file, we get following window. Type “omnetpp” command to open omnet++ IDE.

Step 2: after that command, following window will open.

Step 3: Go to project Explorer > inet > examples > mobility and open MobileNetwork.ned file.

MobileNetwork.ned:

Coding:

MobileNetwork.ned:

package inet.examples.mobility;

import inet.visualizer.integrated.IntegratedCanvasVisualizer;

network MobileNetwork{

parameters:

int numHosts;

bool hasVisualizer = default(false);

@display("bgb=600,400");

submodules:

visualizer: IntegratedCanvasVisualizer if hasVisualizer {

parameters:

@display("p=100,100");

}

host[numHosts]: MobileHost {

parameters:

@display("p=300,200;r=,,#707070");

}

}

Omnetpp.ini:

[General]

#scheduler-class = "inet::cRealTimeScheduler" #so that speed appears realistic

#debug-on-errors = true

sim-time-limit = 10day

*.numHosts = 2

**.constraintAreaMinX = 0m

**.constraintAreaMinY = 0m

**.constraintAreaMinZ = 0m

**.constraintAreaMaxX = 600m

**.constraintAreaMaxY = 400m

**.constraintAreaMaxZ = 0m

**.updateInterval = 0.1s # test with 0s too, and let getCurrentPosition update the display string from a test module

**.mobility.initFromDisplayString = false

[Config AnsimMobility]

network = MobileNetwork

**.host*.mobility.typename = "AnsimMobility"

**.host*.mobility.ansimTrace = xmldoc("ansimtrace.xml")

**.host*.mobility.nodeId = -1  #means "host module's index"

[Config BonnMotionMobility1]

network = MobileNetwork

description = "2 hosts"

**.host*.mobility.typename = "BonnMotionMobility"

**.host*.mobility.traceFile = "bonnmotion_small.movements"

**.host*.mobility.nodeId = -1  #means "host module's index"

[Config BonnMotionMobility2]

network = MobileNetwork

description = "100 hosts"

*.numHosts = 100

**.host*.mobility.typename = "BonnMotionMobility"

**.host*.mobility.traceFile = "bonnmotion_scenario.movements"

**.host*.mobility.nodeId = -1  #means "host module's index"

[Config ChiangMobility]

network = MobileNetwork

*.numHosts = 1

**.host*.mobility.typename = "ChiangMobility"

**.host*.mobility.stateTransitionUpdateInterval = 3s

**.host*.mobility.speed = 10mps

[Config CircleMobility1]

network = MobileNetwork

*.numHosts = 3

**.host*.mobility.typename = "CircleMobility"

**.host*.mobility.cx = 200m

**.host*.mobility.cy = 200m

**.host*.mobility.r = 150m

**.host*.mobility.speed = 40mps

**.host[0].mobility.startAngle = 0deg

**.host[1].mobility.startAngle = 120deg

**.host[2].mobility.startAngle = 240deg

[Config CircleMobility2]

network = MobileNetwork

*.numHosts = 3

**.host*.mobility.typename = "CircleMobility"

**.host[0].mobility.cx = 100m

**.host[1].mobility.cx = 300m

**.host[2].mobility.cx = 500m

**.host*.mobility.cy = 200m

**.host*.mobility.r = 150m

**.host*.mobility.speed = 40mps

**.host*.mobility.startAngle = 0deg

[Config GaussMarkovMobility]

network = MobileNetwork

*.numHosts = 1

**.host*.mobility.typename = "GaussMarkovMobility"

**.host*.mobility.alpha = 0.9

**.host*.mobility.speed = 10mps

**.host*.mobility.angle = 0deg

**.host*.mobility.variance = 40

**.host*.mobility.margin = 30m

[Config LinearMobility]

network = MobileNetwork

**.host*.mobility.typename = "LinearMobility"

**.host*.mobility.initFromDisplayString = false

**.host*.mobility.speed = 50mps

**.host*.mobility.angle = 30deg  # degrees

[Config LinearMobility01]

extends = LinearMobility

**.updateInterval = 0.1s

[Config LinearMobility1]

extends = LinearMobility

**.updateInterval = 1s

[Config LinearMobility10]

extends = LinearMobility

**.updateInterval = 10s

[Config LinearMobility100]

extends = LinearMobility

**.updateInterval = 100s

[Config LinearMobility1000]

extends = LinearMobility

**.updateInterval = 1000s

[Config MassMobility]

network = MobileNetwork

*.numHosts = 5

**.host*.mobility.typename = "MassMobility"

**.host*.mobility.initFromDisplayString = false

**.host*.mobility.changeInterval = truncnormal(2s, 0.5s)

**.host*.mobility.angleDelta = normal(0deg, 30deg)

**.host*.mobility.speed = truncnormal(15mps, 5mps)

[Config MassMobilityWithScenario]

network = MobileNetworkWithScenario

*.numHosts = 5

**.host*.mobility.typename = "MassMobility"

**.host*.mobility.initFromDisplayString = false

**.host*.mobility.changeInterval = truncnormal(2s, 0.5s)

**.host*.mobility.angleDelta = normal(0deg, 30deg)

**.host*.mobility.speed = truncnormal(15mps, 5mps)

**.scenarioManager.script = xmldoc("scenario.xml")

[Config MoBANMobility1]

network = MoBANNetwork

**.constraintAreaMaxX = 1000m

**.constraintAreaMaxY = 1000m

**.constraintAreaMaxZ = 1000m

**.numNodes = 12

**.numMoBAN = 1

**.coordinator[*].postureSpecFile = xmldoc("postures1.xml")

**.coordinator[*].configFile = xmldoc("configMoBAN1.xml")

**.coordinator[*].useMobilityPattern = false

**.coordinator[0].mobilityPatternFile = "MoBAN_Pattern_in0.txt"

**.node[*].mobility.typename = "MoBanLocal"

**.node[*].mobility.coordinatorIndex = 0

[Config MoBANMobility2]

network = MoBANNetwork

**.constraintAreaMaxX = 1000m

**.constraintAreaMaxY = 1000m

**.constraintAreaMaxZ = 1000m

**.numNodes = 24

**.numMoBAN = 2

**.coordinator[*].postureSpecFile = xmldoc("postures1.xml")

**.coordinator[*].configFile = xmldoc("configMoBAN2.xml")

**.coordinator[*].useMobilityPattern = false

**.coordinator[*].mobilityPatternFile = ""

**.node[*].mobility.typename = "MoBanLocal"

**.node[0..11].mobility.coordinatorIndex = 0

**.node[12..23].mobility.coordinatorIndex = 1

[Config RandomWaypointMobility1]

description = "zero waitTime"

network = MobileNetwork

*.numHosts = 5

**.host*.mobility.typename = "RandomWaypointMobility"

**.host*.mobility.initFromDisplayString = false

**.host[0].mobility.speed = 10*uniform(20mps,50mps)

**.host*.mobility.speed = uniform(20mps,50mps)

[Config RandomWaypointMobility2]

description = "nonzero waitTime"

extends = RandomWaypointMobility1

**.host*.mobility.waitTime = uniform(3s,8s)

[Config RectangleMobility]

network = MobileNetwork

**.host*.mobility.typename = "RectangleMobility"

**.host*.mobility.constraintAreaMinX = 100m

**.host*.mobility.constraintAreaMinY = 100m

**.host*.mobility.constraintAreaMaxX = 500m

**.host*.mobility.constraintAreaMaxY = 300m

#**.host*.mobility.x1 = 100

#**.host*.mobility.y1 = 100

#**.host*.mobility.x2 = 500

#**.host*.mobility.y2 = 300

**.host[0].mobility.startPos = 0

**.host[1].mobility.startPos = 2.5

**.host[0].mobility.speed = 20mps

**.host[1].mobility.speed = -10mps

[Config StaticGridMobility]

network = MobileNetwork

*.numHosts = 20

**.host*.mobility.typename = "StaticGridMobility"

**.host*.mobility.marginX = 100m

**.host*.mobility.marginY = 100m

**.host*.mobility.numHosts = 20

[Config StationaryMobility]

network = MobileNetwork

*.numHosts = 3

**.host*.mobility.typename = "StationaryMobility"

# place it at a fixed position:

**.host[0].mobility.initialX = 50m

**.host[0].mobility.initialY = 200m

**.host[0].mobility.initFromDisplayString = false

# the second node is using the display string position (or placed randomly if position is not present in display string)

**.host[1].mobility.initFromDisplayString = true

# place it at a random position:

**.host[2].mobility.initFromDisplayString = false

[Config TractorMobility]

network = MobileNetwork

*.numHosts = 1

**.host*.mobility.typename = "TractorMobility"

**.host*.mobility.x1 = 100m

**.host*.mobility.y1 = 100m

**.host*.mobility.x2 = 500m

**.host*.mobility.y2 = 300m

**.host*.mobility.rowCount = 4

**.host*.mobility.speed = 50mps

[Config TurtleMobility1]

network = MobileNetwork

description = "square"

*.numHosts = 1

**.host*.mobility.typename = "TurtleMobility"

**.host*.mobility.turtleScript = xmldoc("turtle.xml", "movements//movement[@id='1']")

[Config TurtleMobility2]

network = MobileNetwork

description = "two squares"

*.numHosts = 1

**.host*.mobility.typename = "TurtleMobility"

**.host*.mobility.turtleScript = xmldoc("turtle.xml", "movements//movement[@id='2']")

[Config TurtleMobility3]

network = MobileNetwork

description = "random waypoint"

*.numHosts = 2

**.host*.mobility.typename = "TurtleMobility"

**.host*.mobility.turtleScript = xmldoc("turtle.xml", "movements//movement[@id='3']")

[Config TurtleMobility4]

network = MobileNetwork

description = "mass+reflect"

*.numHosts = 2

**.host*.mobility.typename = "TurtleMobility"

**.host*.mobility.turtleScript = xmldoc("turtle.xml", "movements//movement[@id='4']")

[Config TurtleMobility5]

network = MobileNetwork

description = "mass+wrap"

*.numHosts = 2

**.host*.mobility.typename = "TurtleMobility"

**.host*.mobility.turtleScript = xmldoc("turtle.xml", "movements//movement[@id='5']")

[Config TurtleMobility6]

network = MobileNetwork

description = "mass+placerandomly"

*.numHosts = 2

**.host*.mobility.typename = "TurtleMobility"

**.host*.mobility.turtleScript = xmldoc("turtle.xml", "movements//movement[@id='6']")

[Config AttachedMobility]

network = MobileNetwork

description = "attached"

*.numHosts = 3

# mobility visualizer shows velocity and orientation

*.hasVisualizer = true

*.visualizer.mobilityVisualizer.moduleFilter = "**.mobility" # filter for host mobilities, ignore mobility superposition elements

*.visualizer.mobilityVisualizer.displayMovementTrails = true

*.visualizer.mobilityVisualizer.displayOrientations = true

*.visualizer.mobilityVisualizer.displayVelocities = true

# other hosts are also moving around in a larger circle following host[0]

**.host[0].mobility.typename = "CircleMobility"

**.host[0].mobility.cx = 300m

**.host[0].mobility.cy = 200m

**.host[0].mobility.r = 150m

**.host[0].mobility.speed = 40mps

# other hosts are also moving around in a larger circle following host[0]

**.host[*].mobility.typename = "AttachedMobility"

**.host[*].mobility.mobilityModule = "^.^.host[0].mobility"

**.host[1].mobility.offsetX = 50m

**.host[1].mobility.offsetHeading = 90deg

**.host[2].mobility.offsetX = -50m

**.host[2].mobility.offsetHeading = -90deg

[Config SuperpositioningMobility]

network = MobileNetwork

description = "superpositioning"

*.numHosts = 8

# mobility visualizer shows velocity and orientation

*.hasVisualizer = true

*.visualizer.mobilityVisualizer.moduleFilter = "**.mobility" # filter for host mobilities, ignore mobility superposition elements

*.visualizer.mobilityVisualizer.displayMovementTrails = true

*.visualizer.mobilityVisualizer.displayOrientations = true

*.visualizer.mobilityVisualizer.displayVelocities = true

# last host stays in the center of the scene

**.host[7].mobility.typename = "StationaryMobility"

**.host[7].mobility.initialX = 300m

**.host[7].mobility.initialY = 200m

**.host[7].mobility.initialZ = 0m

# other hosts move around the scene using mobility superpositon

**.host[*].mobility.typename = "SuperpositioningMobility"

**.host[*].mobility.numElements = 4

# other hosts are intiallly positioned in static concentric circles

**.host[*].mobility.element[0].typename = "StaticConcentricMobility"

**.host[*].mobility.element[0].subjectModule = "^.^"

**.host[*].mobility.element[0].numHosts = 7

# other hosts are also moving around in a larger circle following host[0]

**.host[0].mobility.element[1].typename = "CircleMobility"

**.host[0].mobility.element[1].faceForward = false

**.host[0].mobility.element[1].cx = 300m

**.host[0].mobility.element[1].cy = 200m

**.host[0].mobility.element[1].r = 150m

**.host[0].mobility.element[1].speed = 40mps

**.host[*].mobility.element[1].typename = "AttachedMobility" # other hosts follow the movement of the 2nd element of host[0]'s mobility superposition

**.host[*].mobility.element[1].mobilityModule = "^.^.^.host[0].mobility.element[1]"

# other hosts are also slightly moving randomly around their position in the group

**.host[*].mobility.element[2].typename = "MassMobility"

**.host[*].mobility.element[2].faceForward = false

**.host[*].mobility.element[2].initFromDisplayString = false

**.host[*].mobility.element[2].initialX = 0m

**.host[*].mobility.element[2].initialY = 0m

**.host[*].mobility.element[2].initialZ = 0m

**.host[*].mobility.element[2].constraintAreaMinX = 0m

**.host[*].mobility.element[2].constraintAreaMinY = 0m

**.host[*].mobility.element[2].constraintAreaMaxX = 50m # limiting the random movement

**.host[*].mobility.element[2].constraintAreaMaxY = 50m # limiting the random movement

**.host[*].mobility.element[2].changeInterval = truncnormal(2s, 1s)

**.host[*].mobility.element[2].angleDelta = normal(0deg, 30deg)

**.host[*].mobility.element[2].speed = truncnormal(10mps, 3mps)

# other hosts are facing towards the last host sitting in the center

**.host[*].mobility.element[3].typename = "FacingMobility"

**.host[*].mobility.element[3].initFromDisplayString = false

**.host[*].mobility.element[3].initialX = 0m

**.host[*].mobility.element[3].initialY = 0m

**.host[*].mobility.element[3].initialZ = 0m

**.host[*].mobility.element[3].sourceMobility = "^" # the superposition is the source

**.host[*].mobility.element[3].targetMobility = "^.^.^.host[7].mobility" # last host is the target

Step 4: Click on Run button.

Step 5: After that following window will open.

Step 6: Click on RUN.

OUTPUT:

Conclusion: We have learnt to create a simple Single mobile network.

Practical No. 08

Aim: Create wireless network in OMNET++.

Step 1: Go to omnetpp-5.5.1 folder in which open “mingwenv” file, we get following window. Type “omnetpp” command to open omnet++ IDE.

Step 2: after that command, following window will open.

Step 3: Go to project Explorer > inet > examples > wireless > crosswalk and open Crosstalk.ned file.

Crosstalk.ned:

Coding:

Crosstalk.ned:

package inet.examples.wireless.crosstalk;

import inet.networklayer.configurator.ipv4.Ipv4NetworkConfigurator;

import inet.node.inet.AdhocHost;

import inet.physicallayer.ieee80211.packetlevel.Ieee80211DimensionalRadioMedium;

import inet.visualizer.contract.IIntegratedVisualizer;

network Crosstalk

{

parameters:

submodules:

visualizer: <default("IntegratedCanvasVisualizer")> like IIntegratedVisualizer if hasVisualizer() {

parameters:

@display("p=100,300;is=s");

}

configurator: Ipv4NetworkConfigurator {

parameters:

@display("p=100,100;is=s");

}

radioMedium: Ieee80211DimensionalRadioMedium {

parameters:

@display("p=100,200;is=s");

}

aNode[2]: AdhocHost {

parameters:

@display("r=,,#707070;p=400,200");

}

bNode[2]: AdhocHost {

parameters:

@display("r=,,#707070;p=250,200");

}

}

Omnetpp.ini:

[General]

network = Crosstalk

#record-eventlog = true

sim-time-limit = 0.1s

seed-set = 1

**.constraintAreaMinX = 0m

**.constraintAreaMinY = 0m

**.constraintAreaMinZ = 0m

**.constraintAreaMaxX = 100m

**.constraintAreaMaxY = 100m

**.constraintAreaMaxZ = 0m

# mobility

*.*Node[*].mobility.typename = "StationaryMobility"

*.*Node[*].mobility.initFromDisplayString = false

# udp App

*.*Node[*].numApps = 1

*.*Node[0].app[0].typename = "UdpSink"

*.*Node[*].app[0].typename = "UdpBasicApp"

*.*Node[*].app[0].localPort = 100

*.*Node[*].app[0].destPort = 100

*.*Node[*].app[0].messageLength = 1250B

*.*Node[*].app[0].startTime = exponential(100us)

*.*Node[*].app[0].sendInterval = exponential(100us)

*.aNode[*].app[0].destAddresses = "aNode[0]"

*.bNode[*].app[0].destAddresses = "bNode[0]"

# nic

*.*Node[*].wlan[*].radio.typename = "Ieee80211DimensionalRadio"

# medium

*.radioMedium.backgroundNoise.dimensions = "time frequency"

# radio

*.*Node[*].wlan[*].radio.transmitter.bandwidth = 20 MHz

*.*Node[*].wlan[*].radio.receiver.bandwidth = 20 MHz

*.*Node[*].wlan[*].radio.transmitter.dimensions = "time frequency"

*.aNode[*].wlan[*].radio.channelNumber = 1

*.bNode[*].wlan[*].radio.channelNumber = ${channelNumber=2..10}

Step 4: Click on Run button.

Step 5: After that following window will open.

Step 6: Click OK and then Click on RUN.

OUTPUT:

Conclusion: We have learnt to wireless network in OMNET++.